Running the Engraver

- After clicking the Print button on your computer, the printer will display that a job is present.

- First set the home position of your art.

  - Click the X/Y Off button

  - Click the Go button to confirm that you want to disable the axes.  The display will now show the coordinates of the laser in the X and Y direction.

  - Click the Pointer button.  This will shine a red colored laser onto your work to aid in the position process.

  - Once you are happy with the home location, click the Set Home button.  The display will confirm that you have set the Home/Center of the work piece.

- Next we’ll set the table height so the laser is properly focused.

  - Click the Stop button (this prevents the laser from moving away from its home position.

  - Click the Focus button

  - Put the gage clip onto the laser so that it is seated on its shelf with the dowels protruding through the clip.

  - Raise the table until the gage clip just touches your part.

- Prepare the laser environment by turning on the air and ventilator

  - To the left of the machine on the wall is an air valve.  Turn this so it is aligned with the pipe.

  - To the right of the machine is the ventilator.  Rotate the knob in a clock-wise direction so the unit turns on (this is at its highest speed).

- Start cutting/engraving

  - Press the Go button when you are all set.  The display will show the user the elapsed time and the resolution of the print.